to:	PRRIP Scoring subcommittee

from:	PRRIP Executive Director’s Office

subject: 	Elwood reservoir groundwater recharge scoring ANALYSIS

date:	August 29, 2018

# EXECUTIVE SUMMARY

The Platte River Recovery Implementation Program (PRRIP or Program) ad-hoc Scoring Subcommittee was formed to advance Water Action Plan (WAP) project scoring towards the Program’s First Increment milestone of reducing shortages to U.S. Fish and Wildlife Service (USFWS) target flows by 50,000 to 70,000 acre-feet per year (AFY). The Scoring Subcommittee recommended a general methodology for calculating project score during the J-2 Regulating Reservoir score discussions, which the Program’s Governance Committee (GC) accepted in 2010. The methodology was followed by the Scoring Subcommittee to score other WAP projects, including the Phelps County Canal Groundwater Recharge project and the Pathfinder Municipal Account Lease project.  The Program’s Executive Director’s Office (EDO) provided technical assistance to the Scoring Subcommittee to score the Elwood Reservoir Groundwater Recharge project. The Program has been recharging water in Elwood Reservoir since 2015.

The Elwood Reservoir recharge project score analyses utilized the general methodology accepted by the GC in 2010 with additional assumptions made for this specific project. The lagged accretions reaching the Platte River from the recharge project were estimated using a site-specific unit response function developed for a locally recalibrated version of the COHYST numerical groundwater model. The analysis splits available excess flow between Program projects and other projects that use excess flows in the Central Nebraska Public Power and Irrigation District’s (CNPPID) system.

During the scoring process, various alternatives described in this memorandum were considered for scoring the Elwood Reservoir recharge project. The Scoring Subcommittee recommends a score for the Elwood Reservoir Groundwater Recharge project of 2,800 AFY for the Program. The recommended score assumes a maximum recharge capacity of 30,000 AF, of which 50 percent (15,000 AF) is allocated to the Program.

# INTRODUCTION

The Program’s ad-hoc Scoring Subcommittee was originally formed in 2010 to advance discussions regarding scoring analyses for proposed WAP projects, at that time specifically for the J-2 Regulating Reservoirs project. The Program’s EDO worked with the Scoring Subcommittee to develop a J-2 Regulating Reservoir Scoring Case Study. Based on the findings of the Case Study, the Scoring Subcommittee proposed a WAP project scoring methodology to the Governance Committee (GC), and the GC approved the recommended methodology in June 2010. The methodology approved by the GC was intended for use in future scoring of WAP projects to maintain consistency in project scoring. However, the Subcommittee and GC also recognized that additional assumptions and variations in the scoring methodology may need to be addressed on a case-by-case basis for other WAP projects.

To the extent possible, the EDO used the previously approved scoring methodology to complete preliminary scoring analyses for the Elwood Reservoir Groundwater Recharge Project. The EDO also identified additional scoring methodology questions requiring input from the Scoring Subcommittee specific to the current project.  The purpose of this memorandum is to provide the Scoring Subcommittee with project scores using various alternatives, document the resolutions to questions pertaining to the scoring methodology, and document the Scoring Subcommittee’s decisions regarding a recommendation for a final project score.

# PROJECT DESCRIPTION

The following sections present an overview of Elwood Reservoir and its use for groundwater recharge by the Program.

## Overview of Elwood Reservoir

Elwood Reservoir is a 37,800 acre-feet (AF) reservoir located in Gosper County, Nebraska, just north of the village of Elwood, Nebraska (see map in Appendix A).  The reservoir was constructed in 1976 and is owned and operated by the Central Nebraska Public Power and Irrigation District (CNPPID) to augment irrigation supplies on the E-65 canal.  It lies just south of the E-65 canal and is filled from the canal via the Carl T. Curtis Pump Station, which consists of three 300-horsepower pumps.  The inflow rate into the reservoir depends on reservoir stage, with the pump efficiency decreasing as the reservoir stage increases and water has to be pumped to a higher elevation.  When all three pumps are operating, inflows range from 275 cubic feet per second (cfs) at lower reservoir stages and about 170 cfs at higher stages.  With only one pump operating the minimum inflow is 75 cfs.  There are no substantial drainages that provide natural inflows into Elwood Reservoir.  Releases from the reservoir flow by gravity back into the E-65 canal, and the reservoir does not release water to any natural surface water drainages.  The reservoir has historically been filled in the spring and fall and released water for irrigation from May through September.

Elwood Reservoir typically operates between a maximum stage elevation of 2,607 feet, corresponding to a reservoir capacity of 37,800 AF, and the minimum operational stage of 2,567 feet as determined by the Carl T. Curtis Pump Station elevation and corresponding to a reservoir capacity of 12,100 AF.  The 12,100 AF below the pump station elevation constitutes the reservoir’s dead pool. While this water is not available for irrigation deliveries, water continues to seep out of the reservoir from the dead pool and it can be used for groundwater recharge purposes.  The reservoir is unlined and the rate of seepage from the reservoir depends on the reservoir stage.

The only other inflows and outflows to or from Elwood Reservoir include precipitation, localized runoff, and evaporation. The nearest weather station to the Elwood Reservoir is located near Smithfield, NE, approximately 8 miles to the southeast.  The station is part of the Automated Weather Data Network (AWDN) weather station network operated by the High Plains Regional Climate Center (HPRCC).  The AWDN station has collected data since June 1995, including precipitation and reference evapotranspiration. Reference evapotranspiration is converted to free water evaporation by applying a factor of 0.91.  The annual average precipitation and evaporation were determined over the June 1995 to March 2018 time period to include in the scoring calculations.  While Elwood Reservoir does not receive any significant inflows from surface water tributaries, it will receive some localized runoff during precipitation events.  The average monthly precipitation volume was increased by 10% to account for localized runoff based on an estimate of the localized drainage area.  The average annual precipitation and runoff at the Smithfield AWDN weather station is 2.07 feet and the average annual free water evaporation is 4.82 feet.  The net of these values (evaporation minus the sum of precipitation and localized runoff) is shown in Figure 1 as the monthly average on a volumetric basis for the maximum area of Elwood Reservoir of 1,200 acres.

Figure 1. Maximum monthly net evaporation (=evaporation – precipitation) at the Smithfield, NE, AWDN weather station

## Groundwater Recharge in Elwood Reservoir

Due to the reservoir’s high seepage rates, the Program entered into a series of Water Service Agreements (WSAs) with CNPPID to divert excess flows into the Elwood Reservoir for intentional groundwater recharge beginning in 2015.  The Program diverted a total of 29,400 AF into Elwood Reservoir for recharge through the end of 2017 under a series of temporary recharge permits from the Nebraska Department of Natural Resources (NDNR) that must be renewed annually.  The most recent WSA and temporary recharge permit documents are included in Appendices B1, B2 and B3.

Over the past several years, the CNPPID has decreased its use of Elwood Reservoir for irrigation purposes from historical levels and expects this reduced usage to continue into the future.  This presents an opportunity for increased use of the reservoir for intentional groundwater recharge.  The CNPPID currently expects to continue using the reservoir to provide approximately 9,000 AF of irrigation deliveries from June through August.  The CNPPID plans to divert approximately 12,250 AF into the reservoir in April to allow for approximately 3,250 AF in losses that will occur between April and when the irrigation water has been fully released in August.

To deliver excess flows to the reservoir for groundwater recharge, excesses are captured at the Tri County Canal Diversion near North Platte, NE, and routed through the CNPPID canal system into the E-65 canal.  Excess flows are then pumped into Elwood Reservoir via the Carl T. Curtis Pump Station.  The stored excess flows seep into the aquifer below Elwood Reservoir over a period of months.  The daily seepage rate depends on reservoir stage, with higher seepage rates occurring at higher stages and decreasing with reservoir stage.  Seepage rates approach 120 AF/day when the reservoir is at its highest stage (2,607 ft, 37,800 AF) and decrease to 30 AF/day when the reservoir level is at or below its effective operational elevation. The average seepage rate from the reservoir is approximately 60 AF/day (30 cfs) over the typical operating range of the reservoir.

Elwood Reservoir lies near the groundwater divide between Republican River and the Platte River.  The aquifer underlying the Elwood Reservoir is primarily made up of quaternary silts, sand and gravel with a hydraulic conductivity ranging from 20 to 100 feet per day.  Some of the water that seeps into the aquifer flows towards the Republican River while most of the water that seeps into the aquifer flows towards the Platte River, with about 76% of the water recharged in Elwood Reservoir eventually ending up in the Platte River.  Water flowing to the Platte River can emerge as baseflow in the Platte River or may be intercepted by Plum Creek, a drainage that flows to the north of the reservoir, and flow to the Platte above ground.  Recharge water travels slowly through the aquifer, with water beginning to arrive at Plum Creek approximately six months after it enters the aquifer or to the Platte as baseflow after about a year.  Recharged water continues to slowly flow into Plum Creek and the Platte River for years after it is recharged.

The general assumptions used in the preliminary scoring analyses by the EDO are described in further detail in Section III. In addition, the EDO identified methodology issues requiring input from the Scoring Subcommittee, which are summarized in Sections IV and V.

# GENERAL METHODOLOGY

The EDO completed preliminary scoring analyses for the Elwood Reservoir Groundwater Recharge project, as shown in the map in Appendix A. The project score ultimately depends upon the Scoring Subcommittee’s recommendations and GC approval.

The scoring analysis consists of two steps, a recharge analysis and a reduction of shortages to target flow analysis.  The recharge analysis evaluates available excesses and Elwood Reservoir operations to determine the amount of water that can be recharged.  The results of the recharge analysis are then used for the subsequent target flow deficit reduction analysis that lags groundwater returns to the river and routes them to Grand Island.  The general scoring methodology and assumptions used for scoring of the Elwood Reservoir Groundwater Recharge project are listed in Table 1.

Table 1. Key Scoring Assumptions

| Component | Data |
| --- | --- |
| Hydrology | OpStudy Adjusted Present Condition with Three State Projects (without pulse flows). EA flows included at Grand Island, but not available for recharge. |
| Analysis Period | 1947-1994 |
| Source of recharged water | Natural flows in excess of USFWS target flows (Excess flows) |
| Recharge Volume Time Step | Monthly (recharge operations) |
| Lagged Accretion Time Step | Monthly (reductions to shortages) |
| Excesses/Shortages Calculation | @ Grand Island |
| Target Flows | Appendix A-5, Column 4 or 8, depending on daily/monthly time step |
| Routing | WMC Loss Model |
| Accretion Modeling Method | Return flow curve developed from numerical model |

Various scoring assumptions were evaluated in the preliminary scoring analysis, resulting in a range of potential scores for the Scoring Subcommittee to consider. For the various alternatives, the EDO completed the preliminary scoring analyses using a monthly time step.

## Reservoir Recharge Analysis

The first step in the Elwood Reservoir groundwater recharge project score analysis involves determining the volume of water that can be recharged in the reservoir on a monthly basis.  Four factors determine the amount of water that can be delivered to the Elwood Reservoir for groundwater recharge, including the available excesses, the capacity of the pumps at the Carl T. Curtis pump station, the annual recharge capacity in Elwood Reservoir, and the storage space available in Elwood Reservoir.

### Available excesses

The source of recharge water for this project is excess flows, or water in the Platte River at Grand Island that is in excess of the USFWS target flows.  The Elwood Reservoir is fed by the E-65 canal which diverts water from the CNPPID’s Tri-County Supply Canal just upstream of the Johnson Reservoir. Therefore, the scoring analysis only considered excess flows available within the CNPPID system as calculated at the J-2 Return. Similar to the Phelps County Canal Groundwater Recharge scoring analysis, hydrology from the OpStudy “Adjusted Present Condition with Three States Projects, without Pulse Flows” model run for 1947 to 1994 was used as the basis for the Elwood Reservoir Groundwater Recharge scoring analysis. Flows at the J-2 Return were used to determine the amount of water available to divert down the E-65 canal.

Flow contributions from Lake McConaughy Environmental Account (EA) releases were included when calculating excesses and shortages to target flows at Grand Island because EA water can influence the amount of shortage that needs to be met by other Program water projects.  However, EA flows estimated to be present in the J-2 Return were excluded from the excess flows available for recharge purposes, which is consistent with the Phelps County Canal scoring analysis. Excess flows identified at Grand Island also had to be physically available in the OpStudy modeling of the J-2 Return in order to be diverted into the E-65 Canal and Elwood Reservoir for recharge purposes. The groundwater recharge project was modeled using a monthly time step due to the delayed nature of groundwater return flows and accretion modeling considerations.

The determination of excess flows available for recharge is based on the greater of the USFWS target flows and instream flow rights held by Central Platte Natural Resources District and Nebraska Game and Parks Commission. A monthly summary of excesses and shortages to target flows at Grand Island is provided in Appendix C.

While it may be possible for the Elwood Reservoir groundwater recharge project to use other sources of water to recharge, such as water from the Lake McConaughy EA, the scoring analysis only considers excess flows.  The amount of water to be recharged cannot exceed the amount of excess flows available at the E-65 canal.

### Excess prioritization

Several projects along the CNPPID canal system compete for excess flows for groundwater recharge or other purposes.  The Program has three WAP projects operating or in development along the CNPPID system to recharge excess flows, including the Phelps County Canal groundwater recharge project, the Cottonwood Ranch broad-scale recharge project, and the Elwood Reservoir groundwater recharge project.  The NDNR and Tri-Basin Natural Resources District (NRD) also have contracts with the CNPPID for recharge in Elwood Reservoir.

The CNPPID outlined its prioritization plan for delivery of excess flows to all of the projects along its canal system in Exhibit F of the WSA for delivery of water to Cottonwood Ranch, included in Appendix B4.  In order to make deliveries to all of the excess flow projects, the Phelps County and E65 canals must divert a minimum of 30 cfs and 10 cfs, respectively.  Any excess flows above these baseline diversions would be divided equally between projects, with the caveat that there must be enough excesses to meet the 75 cfs minimum pumping capacity to divert water into Elwood Reservoir.

### Carl T. Curtis Pump Station Capacity

Another limitation on the amount of water that can be recharged in Elwood Reservoir is the amount of water that can be pumped into the reservoir.  The Carl T. Curtis Pump Station is comprised of three pumps with a maximum capacity of 275 cfs.  The scoring analysis assumed a daily pumping rate of 250 cfs to determine the total monthly volume of water that could physically be pumped into the reservoir for both recharge and irrigation purposes.  This rate approximates the average pumping rate based on the reservoir stage in the scoring model.  Figure 2 below compares the range of constant pumping rates, from 150 cfs to 250 cfs with the average pumping rates based on reservoir stage and CNPPID’s pump curve for three different diversion cap scenarios.  In all cases, the rates are near or above 250 cfs, suggesting this constant value is an appropriate choice for the scoring model.

Figure 2. Calculated vs. constant pumping capacity

The pumping capacity for recharge is limited in months that the CNPPID pumps into the reservoir for irrigation purposes.  Available space in the E-65 Canal also limits the amount of excess flows that can be diverted during the irrigation season.  It is assumed that the canal is filled to the operational capacity of 350 cfs for irrigation from June through August and that no water is available to pump into Elwood Reservoir for Program recharge during those months.

### Annual recharge diversion capacity

The temporary water service agreement between the Program and the CNPPID includes an annual cap on the volume of water that can be diverted for recharge into Elwood Reservoir.  A cap of 8,000 AF was defined in the initial 2015 WSA and increased to 12,000 AF in January 2018 with an amendment (Appendix B2) to the October 2017 WSA (Appendix B1) between the Program and CNPPID for Elwood Reservoir recharge.  The scoring analysis accounts for the cumulative annual recharge for each year and ensures that this does not exceed the annual recharge cap.  A higher cap allows for a greater project score as more water can be recharged each year.  The scoring analysis considers a range of annual recharge diversion caps, from 8,000 AF to 42,000 AF.

### Storage Space available for recharge water

The amount of water that can be recharged in Elwood Reservoir is also constrained by how much storage space remains in the reservoir for any given month.  While in the past the reservoir was filled in the spring and fall and used for irrigation deliveries throughout the summer, the CNPPID intends to reduce its use of the reservoir and only fill it in the spring.  Table 2 presents simulated likely future reservoir operations by CNPPID.  This scoring analysis repeats this expected reservoir operation scheme for every year to represent the CNPPID’s use of the reservoir.  The reservoir can only accommodate water for recharge if there is space in the reservoir in addition to that required by the CNPPID.

Table 2. Simulated CNPPID Annual Reservoir Operations, in thousands of acre-feet (KAF)

| Month | Simulated CNPPID Elwood Inflows | Simulated CNPPID Elwood Releases | Simulated CNPPID Elwood Seepage | Simulated CNNPID Elwood Evap loss | Simulated CNPPID End of Month Storage |
| --- | --- | --- | --- | --- | --- |
| Jan | - | - | - | - | - |
| Feb | - | - | - | - | - |
| Mar | - | - | - | - | - |
| Apr | 12.23 | - | 0.89 | 0.09 | 11.25 |
| May | - | 1.00 | 0.81 | 0.09 | 9.35 |
| Jun | - | 2.00 | 0.65 | 0.07 | 6.63 |
| Jul | - | 3.00 | 0.58 | 0.04 | 3.00 |
| Aug | - | 3.00 | - | - | 0.00 |
| Sep | - | - | - | - | - |
| Oct | - | - | - | - | - |
| Nov | - | - | - | - | - |
| Dec | - | - | - | - | - |
| Annual Total | 12.23 | 9.00 | 2.94 | 0.29 |  |

In addition to the CNPPID’s irrigation water storage in the reservoir, the storage space filled with water to be recharge from previous months factors into reservoir storage calculations.  The end of month reservoir content is compared to the reservoir capacity as well as the upcoming space need by CNPPID for irrigation.  The scoring analysis tracks the amount of CNPPID and recharge water present in the reservoir for each month and accounts for seepage from each pool of water as a share of the total volume.  Seepage rates from the reservoir are determined using the loss curve for Elwood Reservoir provided by the CNPPID.  The seepage rate depends on the reservoir elevation, as shown in Figure 3 (note the inverted y-axis, with zero at the top and reservoir losses increasing downward).

Figure 3. Elwood Reservoir Loss Curve (source: CNPPID)

### Recharge analysis calculations

The calculations of water available for recharge take the limitations discussed above into account by identifying which of the four factors limits the amount of water that can be recharged in a given month.  The calculation used in the analysis determines the minimum of the available excess flow, the monthly total pump capacity at the Carl T. Curtis Pump Station, the difference between the annual recharge cap and the current year’s cumulative recharge, or the current month’s available storage space.  For example, if there are 5,000 AF of available excess flows in a given month but there is only 2,000 AF of available storage space in the reservoir, the volume of water diverted for recharge that month will be 2,000 AF.  If a given month does not have any available excess flows, no water will be recharged for the month.

## Reduction to Target Flow Shortages Analysis

Once the quantity of water to be recharged each month is calculated, the next steps in the scoring analysis are to calculate when and how much of this water arrives at the Platte River as “river accretions,” route the accretions to Grand Island, and determine what portion of the accretions serve to reduce shortages to target flows.

### Lagged accretions at the river

The recharge of the underlying aquifer that results from reservoir seepage eventually enhances flows in the Platte River, and is referred to as the “river accretion.” The timing of river accretions is dependent on aquifer characteristics, the distance between the point of recharge and the river, and the degree to which recharged water is intercepted by intervening surface water features such as Plum Creek. River accretions to the Platte River resulting from recharge in Elwood Reservoir were estimated using a return flow function developed using the MODFLOW groundwater model developed as part of the COHYST integrated model.  The return flow function was developed in conjunction with Hahn Water Resources (Special Advisor) and is described in further detail in Appendix D. The return flow function identified the percentage of recharged water that returns to the Platte River (either directly to the Platte or first to Plum Creek then to as surface flow to the Platte) on a monthly basis.  Monthly volumes of recharged water were passed through the return flow function to determine the quantity and timings of river accretions for the scoring analysis.  The return flow function is shown in Figure 4, with the total percentage of returns shown in red and the corresponding percentage of Platte River and Plum Creek returns shown in green and blue, respectively.

Groundwater drains lying between Elwood Reservoir and the Platte River may also intercept some of the recharged water; however, interception of recharged water by groundwater drains is not expected to have a significant impact on the timing or magnitude of water arriving at the Platte River considering the monthly timestep of the scoring analysis and the close proximity of groundwater drains to the Platte River.

Figure 4. Return flow function for water recharged in Elwood Reservoir.

The percent of water that returns to the Platte River via Plum Creek or directly to the Platte approaches 76%.  The remaining 24% of recharged water eventually flows to the Republican River basin.

### Routing of lagged accretions to Grand Island

The Elwood Reservoir groundwater recharge yield accruing to the river was entered into the preliminary scoring model and routed to Grand Island using transit loss factors derived from the WMC Loss Model. As with the scoring of the Phelps County Canal Groundwater Recharge project, the WMC Loss Model was used to calculate the loss per mile for each month for water years 1975 – 2006. The transit losses were calculated for the Overton to Grand Island reach as all of the recharged water enters the river above Overton.  The transit loss factors were applied to the river accretions to determine the volume of flow reaching Grand Island. On average, approximately 88% of recharge accretions reach Grand Island, or 12% per month is lost in a normal year due to routing. The percentage of water arriving at Grand Island was then averaged by month and year type as shown in Table 3.

Table 3. Average Percentage of River Accretions at Overton Reaching Grand Island, based on WMC Loss Model

| Month | Wet Yr | Normal Yr | Dry Yr |
| --- | --- | --- | --- |
| Jan | 89% | 89% | 85% |
| Feb | 92% | 93% | 91% |
| Mar | 98% | 97% | 95% |
| Apr | 96% | 97% | 95% |
| May | 97% | 95% | 94% |
| Jun | 96% | 93% | 68% |
| Jul | 96% | 79% | 41% |
| Aug | 88% | 77% | 27% |
| Sep | 84% | 74% | 36% |
| Oct | 91% | 84% | 54% |
| Nov | 92% | 87% | 74% |
| Dec | 92% | 88% | 85% |
| Avg | 93% | 88% | 70% |

### Reductions to target flow shortages

River accretions arriving at Grand Island are used in conjunction with monthly target flows (listed in Column 8 of Appendix A-5 of the Program Document, a monthly summary of excesses and shortages to target flows at Grand Island is provided in Appendix C) and the monthly river flow at Grand Island from the OpStudy Hydrology (“Adjusted Present Condition with Three States Projects, without Pulse Flows” model run for 1947 to 1994).  River accretions arriving at Grand island when the Grand Island monthly flow volume is below the monthly target flow volume counted as reducing deficits while accretions arriving when Grand Island monthly flow volumes exceeded the monthly flow volume did not.  The projects score is determined by summing the volume of river accretions that reduced deficits to target flows on an annual basis and averaging the annual totals over the 1947-1994 analysis period.

# OTHER SCORING ASSUMPTIONS

The EDO requested that the Scoring Subcommittee review the assumptions and policy considerations outlined in this section and provide input to the EDO. The Scoring Subcommittee’s recommendations were used to finalize the Elwood Reservoir groundwater recharge scoring analysis, and ultimately used to assign a final project score contribution towards meeting the Program’s First Increment Water Objective. In addition, the methodology considerations addressed by the Subcommittee may be incorporated into the scoring of future WAP projects.

## CNPPID Operations

The assumptions underlying the simulated CNPPID operations of Elwood Reservoir impact the storage space in the reservoir that is available for recharge use and thus the project score.  The current simulated operational scheme does not vary from year to year to account for varying irrigation demands during wet or dry years.  The operational scheme does not account for future increased or decreased use of the reservoir for irrigation purposes.

## Sharing Water with Program Partners

As noted in the WAP, a Nebraska-based Program partner, such as Tri-Basin NRD, may use a portion of the river accretions resulting from groundwater recharge for addressing Nebraska New Depletion Plan (NNDP) requirements. Both the Tri-Basin NRD and the NDNR have entered into separate water service agreements with the CNPPID to recharge water in Elwood Reservoir, with their combined total recharge varying from less than 3,000 AF annually up to 15,000 AF, with an annual average of approximately 7,500 AF.

As use of Elwood Reservoir for groundwater recharge increases, the Program will work with its partners to develop an equitable approach to sharing the available space for groundwater recharge in the reservoir.  This may take the form a joint water service agreement between the Program and its partners or separate agreements for each entity.  If separate water service agreements are pursued, the annual recharge diversion cap will limit the amount of water the Program can recharge in the reservoir and ensure that other entities have access to available space in the reservoir.  While the Program’s current WSA with CNPPID for Elwood Reservoir groundwater recharge specifically caps billings to the Program at 12,000 AFY (without consent from the Program to exceed), the Scoring Subcommittee assumed 50 percent of the total recharge capacity is available to the Program.

## Annual Recharge Diversion Cap

The annual cap on diversions for recharge set in the most recent water service agreement with the CNPPID is 12,000 AF.  However, the CNPPID intends to decrease its use of Elwood Reservoir for irrigation purposes, which may allow for future increases to the diversion cap.  The CNPPID currently estimates an annual 30,000 AF of space will be available for recharge water.  The Program would split this volume with the NDNR and the Tri-Basin NRD, resulting in an annual diversion cap of 15,000 AF for the Program.  Because this diversion cap may change as the CNPPID’s use of Elwood evolves and the NDNR and Tri-Basin NDR refine their recharge needs, scoring analyses were performed using a range of diversion caps.

# ALTERNATIVES & SCORES

The EDO completed a scoring analysis based on the assumptions described in previous sections.  A range of recharge diversion caps was evaluated as shown in Figure 5, which also shows the portion of the score (50 percent) that would be available for the Program to use for recharge.

Figure 5. Elwood recharge diversion caps and corresponding project scores

The project score increases in a roughly linear fashion over the range of recharge diversion caps of 8,000 AF to 22,000 AF then flattens off at higher diversion caps. In this range, the recharge diversion cap has a direct relationship to project score, with higher caps resulting in higher scores.  At higher recharge caps, the availability of excess flows becomes a limiting factor for the score.

Score estimates for select recharge cap are presented in Table 4; values shown represent the annual average over the 48-year analysis period for each scenario.  These scenarios include the following: the current 12,000 AF (a total cap of 24,000 AF), a decrease of 4,000 AF to the previous diversion cap of 8,000 AF (a total cap of 16,000 AF), an increase of 3,000 AF to a cap of 15,000 AF (a total cap of 30,000 AF), and an increase of 8,000 AF to a cap of 20,000 AF (a total cap of 40,000).  It is assumed that the Program would utilize half of the total annual recharge diversion cap while the remaining half would be used by the NDNR and the Tri-Basin NRD.

Table 4. Estimated Project Scores at Grand Island

| Scenario | Total Annual Avg Recharged Volume (AF) | Program Annual Avg Recharged Volume (AF) | Total Score (AF) | Program Score (AF) |
| --- | --- | --- | --- | --- |
| Recharge cap of 8,000 AF (total cap of 16,000 AF) | 13,000 | 6,500 | 3,800 | 1,900 |
| Current WSA: annual recharge diversion cap of 12,000 AF (a total cap of approximately 24,000 AF) | 17,400 | 8,700 | 5,200 | 2,600 |
| Recharge cap of 15,000 AF (total cap of 30,000 AF) | 18,700 | 9,350 | 5,600 | 2,800 |
| Recharge cap of 20,000 AF (total cap of 40,000) | 19,100 | 9,550 | 5,700 | 2,850 |

Detailed score tables by month for annual recharge diversion caps corresponding to Table 4 are included in Appendix E. The difference between the diverted volume and the recharged volume is due to evaporation losses while recharged water is stored in Elwood Reservoir.  The differences between the recharged volume and the returns to the river in Table 4 are due in part to approximately 24% of the recharged water flowing to the Republican River basin (117,000 AF of the 480,000 AF recharged over the 48 year period) and in part because approximately 69,000 AF (14% of the 480,000 AF recharged over the 48 year period) of recharged water remains in storage at the end of the 48-year analysis period.  The 69,000 AF in storage will eventually accrue to the Platte River over a longer time period.

# ADDITIONAL CONSIDERATIONS FOR DISCUSSION

The following additional considerations for the Elwood Reservoir groundwater recharge score analysis were addressed in discussions with the Scoring Subcommittee.

## Recapture of Recharged Water

Of the water that is diverted into Elwood Reservoir for groundwater recharge, less than 30% returns to the river at times of shortage to target flows.  Adding groundwater wells that pump recharged water back to the river could increase the project score in several ways.  First, it would allow the Program to control when recharged water returns to the river to maximize the project’s ability to reduce shortages to target flows.  Pumping recharged water would also allow the Program to capture the 24% of recharged water that eventually flows to the Republican River basin.  Additionally, the Program could access the large amount of recharged water that remains in storage (69,000 AF of recharged water remains in storage at the end of the 48-year analysis period) during especially dry time or times of greater shortages to target flows.  The EDO reported that staff are beginning to evaluate options to develop networks of recapture wells to enhance several Program recharge projects on the south side of the Platte River, within the Tri-Basin NRD.

## Lagged Accretions After the Study Period

During the July 26, 2018 (see minutes in Appendix F) conference call, the Scoring Subcommittee requested that the EDO evaluate options for dealing with lagged accretions that occur after the end of the 1947-1994 study period.  The EDO reported back to the Scoring Subcommittee on August 27, 2018 (see minutes in Appendix G) with several alternatives for including lagged accretions that occur after the end of the 1947-1994 study period into the scoring model.

Approximately 69,000 AF of water remains in the aquifer and will eventually arrive at the Platte River after the end of the score model’s analysis period.  The Scoring Subcommittee discussed the merits of adding the lagged accretions back into the model and decided not to take this approach.  Rather, it was noted that the lagged accretions may be the source water for future Program recapture well WAP projects.  The Scoring Subcommittee said it would be more appropriate for the lagged accretions to be reflected in the scoring of recapture well projects than to simply incorporate the remaining volume mathematically into the Elwood recharge score.

The Elwood Reservoir recharge project score could also be modified in the future to account for a higher level of consistent returns after operations have continued for long enough to reach more of a steady state

## Historical Elwood Reservoir Seepage

During the July 26, 2018 Scoring Subcommittee conference call (see minutes in Appendix F), it was requested that the EDO take a coarse look at historical Elwood Reservoir seepage and how differences between that and assumptions made in the score model could affect the project score.

The EDO conducted an analysis that assumed that seepage and resulting baseflows from historic Elwood Reservoir operations from 1978 to 1994 were reflected in the OPSTUDY hydrology.  The difference in historic baseflows and baseflows in the scoring model Elwood Reservoir operations were removed from the OPSTUDY hydrology.  Removing baseflows resulted in decreasing excesses available for diversion and increasing shortages at Grand Island.  The net result was a score increase of 900 AF for a recharge cap of 12,000 AF.

The Scoring Subcommittee discussed the issue in depth during the August 27, 2018 conference call (see minutes in Appendix G).  It was agreed this issue is fundamentally a policy question in nature.  It would reflect a departure from the existing scoring methodology and would have future implications as similar changes to the river’s baseflow occur.  The Scoring Subcommittee decided not to recommend to the governance committee a score that included adjustments to account for historical Elwood Reservoir seepage.

# CONCLUSIONS

Based on current agreements and operations (12,000 AF recharge cap for the Program) and the assumptions outlined in this document, the Elwood Reservoir groundwater recharge project would have a score of 2,600 AFY.  However, it is reasonably assumed that a total recharge capacity of 30,000 AF will be available in Elwood Reservoir, with 50 percent (15,000 AF) allocated to the Program.  Per Figure 5 and Table 4, this would provide a total score of 5,600 AFY, or 2,800 AFY for the Program.  The Scoring Subcommittee recommends a score of 2,800 AFY for this project.

# APPENDIX A

# Elwood Reservoir Location Map

# APPENDIX B

B1.  Water Service Agreement-Groundwater Recharge from Excess Flows Between The Central Nebraska Public Power and Irrigation District, Nebraska Community Foundation and Platte River Recovery Implementation Program.

October 20, 2017

B2.  Amendment No. 1 to the Water Service Agreement-Groundwater Recharge from Excess Flows Between The Central Nebraska Public Power and Irrigation District, Nebraska Community Foundation and Platte River Recovery Implementation Program.

January 8, 2018

B3.  State of Nebraska Department of Natural Resources.  Approval of Application A-19559, Water Division 1-A.

September 25, 2017

B4.  Exhibit F from DRAFT Water Service Agreement (for Diversion Project to Deliver Water from Phelps County Canal to Cottonwood Ranch).

Revision Date May 29, 2018

# APPENDIX C

# Monthly Summary of Excess and Shortage to Target Flows at Grand Island

# APPENDIX D

# Memorandum:  Analysis of Return Flows Associated with Seepage from Elwood Reservoir

Hahn Water Resources (Revised May 18, 2018)

# APPENDIX E

# Score Summary Tables

APPENDIX F

Scoring Subcommittee Conference Call Minutes (July 26, 2018)

APPENDIX G

Scoring Subcommittee Conference Call Minutes (August 27, 2018)

The remaining Elwood Reservoir recharge capacity is allocated to the Nebraska Department of Natural Resources and the Tri-Basin Natural Resources District.

“Water Action Plan Project Scoring Case Study:  CNPPID Reregulating Reservoir” dated April 22, 2010 by the ED Office.

Memo from Scoring Subcommittee to GC regarding “CNPPID Reregulating Reservoir Scoring Recommendation” dated May 12, 2010.

See June 2010 GC Meeting Minutes.

Funk Lagoon Water Action Plan Project Feasibility Report. 09/16/2015.  Prepared by ED Office of the PRRIP.  Draft for Review by the WAC.

Tipping bucket style precipitation gages do not capture all solid (snow) precipitation, thus reported precipitation in winter months may be slightly lower than actually occurred.

Personal correspondence with Cory Steinke of the CNPPID on 3/22/2018

Personal correspondence with Cory Steinke of the CNPPID on 3/22/2018

Cannia, J., Woodward, D., and Cast, L. Cooperative Hydrology Study (COHYST) Hydrostratographic Units and Aquifer Characterization Report. 2/24/2006

Based on Column 4 or 8 of Appendix A-5 in Attachment 5 of the Program Document. Column 4 = target flows in “cfs” were used for the daily analysis. Column 8 = target flows in “average cfs” were summed on a monthly basis and converted to acre-feet as a monthly target flow volume in the scoring model.

WMC Loss Model is referenced in the Water Management Study (2008) by Boyle Engineering Corporation.

COHYST 2010 MODFLOW model with localized recalibration by Hahn Water Resources, ED Office Special Advisor.

EA flow in the J2 Return is not an OpStudy output. Monthly values were estimated by subtracting the EA volume at Overton from the EA volume at Cozad.  The difference was assumed to have been returned to the river through the J2 Return.

Based on personal correspondence with Cory Steinke of the CNPPID on 3/22/2018

COHYST 2010: A Total Water Budget Approach to Integrated Water Management in the Platte River Nebraska. June 2017.  cohyst.nebraska.gov

The yield refers to the amount of recharge at the river. Note that the score is the portion of the yield routed to Grand Island

that occurs during shortages to USFWS target flows.

Amendment No. 1 to the October 20, 2017 WSA, dated January 8, 2018